# Ammonia Bunkering Infrastructure Optimization

## Case 2-2: Ulsan → Busan

| Case ID | case_2_ulsan |
| --- | --- |
| Report Date | 2025-11-17 |
| Time Period | 2030-2050 |
| Analysis Tool | Green Corridor MILP Optimizer v2.0 |

# Executive Summary

This report presents the optimization results for Case 2-2: Ulsan → Busan.
The analysis evaluates 64 different configurations of shuttle vessels and pump systems
over a 20-year period
(2030-2050) to minimize the Net Present Cost (NPC).

## Key Metrics

| Total Feasible Scenarios | 64 |
| --- | --- |
| Optimal Shuttle Size (m³) | 5000 |
| Optimal Pump Flow (m³/h) | 2000 |
| Minimum NPC (M USD) | 119.26 |
| Annual Demand (2050, m³) | 19,049,956 |

# Case Description

## Route Information

Source: Ulsan

Destination: Busan

Distance: 25.0 nautical miles

Travel Time: 1.67 hours

## Operational Parameters

| Shuttle Sizes Available (m³) | 5000, 10000, 15000, 20000, 25000... |
| --- | --- |
| Pump Flow Rates (m³/h) | 400, 600, 800, 1000, 1200... |
| Tank Storage | Disabled |
| Tank Size (tons) | 35000.0 |
| Setup Time (hours) | 0.5 |
| Max Annual Hours | 8000.0 |

# Optimal Solution

The optimization identifies Shuttle Size 5000 m³ with Pump Flow 2000 m³/h as the optimal configuration.

## Cost Breakdown

| Total NPC (M USD) | 119.26 |
| --- | --- |
| Shuttle CAPEX (M USD) | 55.22 |
| Bunkering CAPEX (M USD) | 4.37 |
| Terminal CAPEX (M USD) | 0.00 |
| Shuttle fOPEX (M USD) | 25.50 |
| Bunkering fOPEX (M USD) | 2.02 |
| Shuttle vOPEX (M USD) | 25.12 |
| Bunkering vOPEX (M USD) | 7.05 |
| Terminal vOPEX (M USD) | 0.00 |

# Scenario Analysis

This section presents the top scenarios ranked by Net Present Cost (NPC).

## Top 10 Scenarios

| Rank | Shuttle (m³) | Pump (m³/h) | NPC (M USD) | CAPEX (M USD) | OPEX (M USD) |
| --- | --- | --- | --- | --- | --- |
| 1 | 5000 | 2000 | 119.26 | 59.58 | 59.67 |
| 2 | 5000 | 1800 | 126.02 | 64.82 | 61.20 |
| 3 | 5000 | 1600 | 130.26 | 67.17 | 63.09 |
| 4 | 5000 | 1400 | 137.72 | 72.79 | 64.93 |
| 5 | 10000 | 2000 | 140.22 | 80.87 | 59.35 |
| 6 | 5000 | 1200 | 147.19 | 79.53 | 67.66 |
| 7 | 10000 | 1800 | 148.83 | 85.71 | 63.12 |
| 8 | 10000 | 1600 | 158.07 | 93.30 | 64.77 |
| 9 | 5000 | 1000 | 160.33 | 88.34 | 71.99 |
| 10 | 15000 | 2000 | 160.72 | 96.35 | 64.37 |

# Cost Component Analysis

For the optimal solution, the total cost is dominated by CAPEX (50.0%) and OPEX (50.0%).

| Capital Expenditure (CAPEX) | 50.0% |
| --- | --- |
| Operating Expenditure (OPEX) | 50.0% |
| CAPEX Amount (M USD) | 59.58 |
| OPEX Amount (M USD) | 59.67 |

# Appendix: Raw Data Summary

Total scenarios evaluated: 64

Time period: 2030 - 2050

## Summary Statistics

| Minimum NPC (M USD) | 119.26 |
| --- | --- |
| Maximum NPC (M USD) | 442.95 |
| Average NPC (M USD) | 260.93 |
| Median NPC (M USD) | 260.20 |
| Std Dev NPC (M USD) | 82.30 |

For complete data including yearly breakdowns, refer to the accompanying Excel file.