# Ammonia Bunkering Infrastructure Optimization

## Case 2-1: Yeosu → Busan

| Case ID | case_2_yeosu |
| --- | --- |
| Report Date | 2025-11-17 |
| Time Period | 2030-2050 |
| Analysis Tool | Green Corridor MILP Optimizer v2.0 |

# Executive Summary

This report presents the optimization results for Case 2-1: Yeosu → Busan.
The analysis evaluates 62 different configurations of shuttle vessels and pump systems
over a 20-year period
(2030-2050) to minimize the Net Present Cost (NPC).

## Key Metrics

| Total Feasible Scenarios | 62 |
| --- | --- |
| Optimal Shuttle Size (m³) | 5000 |
| Optimal Pump Flow (m³/h) | 2000 |
| Minimum NPC (M USD) | 158.22 |
| Annual Demand (2050, m³) | 19,049,956 |

# Case Description

## Route Information

Source: Yeosu

Destination: Busan

Distance: 86.0 nautical miles

Travel Time: 5.73 hours

## Operational Parameters

| Shuttle Sizes Available (m³) | 5000, 10000, 15000, 20000, 25000... |
| --- | --- |
| Pump Flow Rates (m³/h) | 400, 600, 800, 1000, 1200... |
| Tank Storage | Disabled |
| Tank Size (tons) | 35000.0 |
| Setup Time (hours) | 0.5 |
| Max Annual Hours | 8000.0 |

# Optimal Solution

The optimization identifies Shuttle Size 5000 m³ with Pump Flow 2000 m³/h as the optimal configuration.

## Cost Breakdown

| Total NPC (M USD) | 158.22 |
| --- | --- |
| Shuttle CAPEX (M USD) | 68.94 |
| Bunkering CAPEX (M USD) | 5.45 |
| Terminal CAPEX (M USD) | 0.00 |
| Shuttle fOPEX (M USD) | 31.22 |
| Bunkering fOPEX (M USD) | 2.47 |
| Shuttle vOPEX (M USD) | 43.09 |
| Bunkering vOPEX (M USD) | 7.05 |
| Terminal vOPEX (M USD) | 0.00 |

# Scenario Analysis

This section presents the top scenarios ranked by Net Present Cost (NPC).

## Top 10 Scenarios

| Rank | Shuttle (m³) | Pump (m³/h) | NPC (M USD) | CAPEX (M USD) | OPEX (M USD) |
| --- | --- | --- | --- | --- | --- |
| 1 | 5000 | 2000 | 158.22 | 74.40 | 83.82 |
| 2 | 5000 | 1800 | 161.94 | 76.45 | 85.49 |
| 3 | 10000 | 2000 | 164.02 | 87.87 | 76.15 |
| 4 | 5000 | 1600 | 168.68 | 81.66 | 87.03 |
| 5 | 5000 | 1400 | 171.90 | 83.41 | 88.48 |
| 6 | 10000 | 1800 | 174.60 | 96.22 | 78.37 |
| 7 | 10000 | 1600 | 179.77 | 99.11 | 80.66 |
| 8 | 5000 | 1200 | 183.00 | 91.07 | 91.93 |
| 9 | 15000 | 2000 | 187.07 | 109.55 | 77.52 |
| 10 | 5000 | 1000 | 194.91 | 99.18 | 95.73 |

# Cost Component Analysis

For the optimal solution, the total cost is dominated by CAPEX (47.0%) and OPEX (53.0%).

| Capital Expenditure (CAPEX) | 47.0% |
| --- | --- |
| Operating Expenditure (OPEX) | 53.0% |
| CAPEX Amount (M USD) | 74.40 |
| OPEX Amount (M USD) | 83.82 |

# Appendix: Raw Data Summary

Total scenarios evaluated: 62

Time period: 2030 - 2050

## Summary Statistics

| Minimum NPC (M USD) | 158.22 |
| --- | --- |
| Maximum NPC (M USD) | 432.60 |
| Average NPC (M USD) | 278.65 |
| Median NPC (M USD) | 280.01 |
| Std Dev NPC (M USD) | 73.99 |

For complete data including yearly breakdowns, refer to the accompanying Excel file.