# Ammonia Bunkering Infrastructure Optimization

## Case 1: Busan Port with Storage

| Case ID | case_1 |
| --- | --- |
| Report Date | 2025-11-20 |
| Time Period | 2030-2050 |
| Analysis Tool | Green Corridor MILP Optimizer v2.0 |

# Executive Summary

This report presents the optimization results for Case 1: Busan Port with Storage.
The analysis evaluates 90 different configurations of shuttle vessels and pump systems
over a 20-year period
(2030-2050) to minimize the Net Present Cost (NPC).

## Key Metrics

| Total Feasible Scenarios | 90 |
| --- | --- |
| Optimal Shuttle Size (m³) | 2500 |
| Optimal Pump Flow (m³/h) | 2000 |
| Minimum NPC (M USD) | 176.74 |
| Annual Demand (2050, m³) | 30,000,000 |

# Case Description

## Operational Parameters

| Shuttle Sizes Available (m³) | 500, 1000, 1500, 2000, 2500... |
| --- | --- |
| Pump Flow Rates (m³/h) | 400, 600, 800, 1000, 1200... |
| Tank Storage | Enabled |
| Tank Size (tons) | 35000.0 |
| Setup Time (hours) | 0.5 |
| Max Annual Hours | 8000.0 |

# Optimal Solution

The optimization identifies Shuttle Size 2500 m³ with Pump Flow 2000 m³/h as the optimal configuration.

## Cost Breakdown

| Total NPC (M USD) | 176.74 |
| --- | --- |
| Shuttle CAPEX (M USD) | 49.99 |
| Bunkering CAPEX (M USD) | 5.63 |
| Terminal CAPEX (M USD) | 53.51 |
| Shuttle fOPEX (M USD) | 22.60 |
| Bunkering fOPEX (M USD) | 2.54 |
| Shuttle vOPEX (M USD) | 20.59 |
| Bunkering vOPEX (M USD) | 5.55 |
| Terminal vOPEX (M USD) | 1.21 |

# 운항 시간 분석 (Time Structure Analysis)

## 최적 시나리오의 운항 시간 구성

최적 시나리오 (Shuttle 2500 m³, Pump 2000 m³/h)의 1회 왕복 운항 시간을 상세히 분석합니다. 총 사이클 시간은 6.92시간이며, 이는 연간 최대 1157회의 운항을 가능하게 합니다.

### 시간 구성 요소 분해

| 시간 구성 요소 | 시간 (h) | 비율 (%) |
| --- | --- | --- |
| 육상 연료 적재 | 1.67 | 31.7% |
| 편도 항해 | 1.00 | 19.0% |
| 호스 연결 및 퍼징 | 1.00 | 19.0% |
| 벙커링 펌핑 | 1.25 | 23.8% |
| 호스 분리 및 퍼징 | 1.00 | 19.0% |
| 복귀 항해 | 1.00 | 19.0% |
| 【총 사이클 시간】 | 6.92 | 100% |

### 연간 운영 지표

| 연간 최대 항차 | 1157 회 |
| --- | --- |
| 연간 공급 용량 | 2891566 m³ |
| 시간 활용도 | 100.0% |
| 셔틀당 평균 일정 | 0.3 일 |

### 시간 분석

벙커링 펌핑이 전체 사이클의 31.7%를 차지하여, 펌프 크기 선택이 가장 중요한 최적화 포인트입니다. 기본 사이클 시간(육상 제외)은 5.25시간이며, 육상 연료 적재에 추가로 1.67시간이 소요됩니다. 이 시간 구조는 연간 1157회의 운항을 가능하게 하며, 총 20년간 약 57.8백만 m³의 암모니아 연료를 공급할 수 있습니다.

# Scenario Analysis

This section presents the top scenarios ranked by Net Present Cost (NPC).

## Top 10 Scenarios

| Rank | Shuttle (m³) | Pump (m³/h) | NPC (M USD) | CAPEX (M USD) | OPEX (M USD) |
| --- | --- | --- | --- | --- | --- |
| 1 | 2500 | 2000 | 176.74 | 109.13 | 67.61 |
| 2 | 2500 | 1800 | 178.73 | 110.30 | 68.42 |
| 3 | 2500 | 1600 | 179.51 | 110.67 | 68.83 |
| 4 | 2500 | 1400 | 184.86 | 114.64 | 70.22 |
| 5 | 2500 | 1200 | 186.43 | 115.58 | 70.85 |
| 6 | 1000 | 1600 | 187.90 | 95.55 | 92.34 |
| 7 | 1000 | 1800 | 188.38 | 95.96 | 92.42 |
| 8 | 1000 | 1200 | 188.85 | 96.28 | 92.56 |
| 9 | 1000 | 2000 | 188.97 | 96.43 | 92.54 |
| 10 | 1000 | 1400 | 189.17 | 96.60 | 92.56 |

# Cost Component Analysis

For the optimal solution, the total cost is dominated by CAPEX (61.7%) and OPEX (38.3%).

| Capital Expenditure (CAPEX) | 61.7% |
| --- | --- |
| Operating Expenditure (OPEX) | 38.3% |
| CAPEX Amount (M USD) | 109.13 |
| OPEX Amount (M USD) | 67.61 |

# Appendix: Raw Data Summary

Total scenarios evaluated: 90

Time period: 2030 - 2050

## Summary Statistics

| Minimum NPC (M USD) | 176.74 |
| --- | --- |
| Maximum NPC (M USD) | 504.52 |
| Average NPC (M USD) | 244.90 |
| Median NPC (M USD) | 231.09 |
| Std Dev NPC (M USD) | 60.08 |

For complete data including yearly breakdowns, refer to the accompanying Excel file.