# Chapter 1: Executive Summary

## Key Findings

The MILP optimization identifies the most cost-effective ammonia bunkering configurations for Busan Port across three supply scenarios. Case 1 (local storage) is the most economical, followed by Case 2-2 (Ulsan), with Case 2-1 (Yeosu) being the most expensive due to longer transport distances.

## Optimal Configurations (1000 m3/h Pump)

| Case | Configuration | NPC (20yr) | LCOAmmonia | Fleet Size (2050) |
| --- | --- | --- | --- | --- |
| Case 1: Busan Storage | 2,500 m3 shuttle | $237.05M | $1.01/ton | ~20 shuttles |
| Case 2-1: Yeosu | 10,000 m3 shuttle | $747.18M | $3.17/ton | ~10 shuttles |
| Case 2-2: Ulsan | 5,000 m3 shuttle | $402.37M | $1.71/ton | ~15 shuttles |

## Cost Comparison

Case 1 (Busan)  : ████████████████████ $237M (baseline)
Case 2-2 (Ulsan): ████████████████████████████████████ $402M (+70%)
Case 2-1 (Yeosu): ████████████████████████████████████████████████████████████████ $747M (+215%)

## Why Case 1 is Optimal

- Short Travel Distance: 1 hour round trip vs 3.34h (Ulsan) or 11.46h (Yeosu)

- Higher Utilization: More cycles per year = better asset utilization

- Smaller Shuttles: 2,500 m3 vs 5,000-10,000 m3 = lower CAPEX per unit

## Why 2,500 m3 Shuttle (Case 1)

| Shuttle Size | NPC | LCO | Reason |
| --- | --- | --- | --- |
| 2,000 m3 | $281.7M | $1.20/ton | Higher OPEX (more trips) |
| 2,500 m3 | $237.05M | $1.01/ton | Optimal balance |
| 3,000 m3 | $282.25M | $1.20/ton | Higher CAPEX |
| 5,000 m3 | $264.24M | $1.12/ton | Overkill for demand |

The 2,500 m3 shuttle requires 2 trips per bunkering call (5000 m3 demand), achieving optimal balance between: - Fleet size (capital cost) - Trip frequency (operating cost) - Asset utilization (efficiency)

## Recommendation

For Busan Port ammonia bunkering infrastructure (2030-2050):

- Primary Recommendation: Build local storage tanks at Busan Port with 2,500 m3 shuttle fleet

  - 20-year NPC: $237.05M

  - Levelized cost: $1.01/ton

- Alternative (if local storage not feasible): Ulsan supply with 5,000 m3 shuttle fleet

  - 20-year NPC: $402.37M

  - Levelized cost: $1.71/ton

  - Premium: +70% over Case 1

- Not Recommended: Yeosu supply

  - 20-year NPC: $747.18M

  - Levelized cost: $3.17/ton

  - Premium: +215% over Case 1

## Key Assumptions

- Pump Rate: 1000 m3/h (fixed for main analysis)

- Discount Rate: 0.0 (no time value of money adjustment)

- Annualization Rate: 7.0% (for asset cost spreading)

- Demand Growth: 50 vessels (2030) to 500 vessels (2050)

- Bunker Volume: 5,000 m3 per call